| <small>SREE VALLIAPPA TEXTILES LTD</small> <small>YARN SPECIFICATIONS</small> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| <small>COTTON COUNTS:</small> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| <small> </small> |  |  |  |  |  |  |  |  |  |  |  |  |  |  | <small>USTER Results per 1000meters(1 Kilo Meter KM)</small> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| <small>Ne</small> | <small>Ply</small> | <small>Nm</small> | <small>Material</small> | <small>BLEND</small> | <small>Spinning</small> | <small>Enduse</small> | <small>CountNe </small> | <small>U%</small> | <small>THIN</small> | <small>THICK</small> | <small>NEPS</small> | <small>total</small> | <small>RKm</small> | <small>CSP</small> | <small>Elongation%</small> | <small>Tpm</small> | <small>tm</small> |
| <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small>Type</small> | <small> </small> | <small>CV</small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>100</small> | <small>1</small> | <small> </small> | <small>100% cotton Combed</small> | <small>100%</small> | <small>Ring</small> | <small>weaving</small> | <small>2.07</small> | <small>14.3</small> | <small>40</small> | <small>309</small> | <small>189</small> | <small>538</small> | <small>17.00</small> | <small> </small> | <small> </small> | <small>1496</small> | <small>3.8</small> |
| <small>100</small> | <small>2</small> | <small> </small> | <small>100% cotton Combed</small> | <small>100%</small> | <small>Ring/Ring DbldTfo</small> | <small>High Twist</small> | <small>1.85</small> | <small>14.41</small> | <small>22</small> | <small>69</small> | <small>48</small> | <small>139</small> | <small>15.33</small> | <small> </small> | <small> </small> | <small>1531</small> | <small>5.5</small> |
| <small>100</small> | <small>2</small> | <small> </small> | <small>100% cotton Combed</small> | <small>100%</small> | <small>Ring/Ring DbldTfo</small> | <small>Gassed n merserised</small> | <small>To be updated</small> |  |  | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small>1058</small> | <small>3.8</small> |
| <small>94</small> | <small>2</small> | <small> </small> | <small>100% cotton Combed</small> | <small>100%</small> | <small>Ring/Ring DbldTfo</small> | <small>weaving</small> | <small>1.73</small> | <small>13.21</small> | <small>19</small> | <small>78</small> | <small>53</small> | <small>150</small> | <small>18.90</small> | <small> </small> | <small> </small> | <small>972</small> | <small>3.6</small> |
| <small>84</small> | <small>2</small> | <small> </small> | <small>100% cotton Combed</small> | <small>100%</small> | <small>Ring/Ring DbldTfo</small> | <small>weaving</small> | <small>1.95</small> | <small>12.96</small> | <small>17</small> | <small>53</small> | <small>71</small> | <small>141</small> | <small>19.00</small> | <small> </small> | <small> </small> | <small>919</small> | <small>3.6</small> |
| <small>74</small> | <small>2</small> | <small> </small> | <small>100% cotton Combed</small> | <small>100%</small> | <small>Ring/Ring DbldTfo</small> | <small>weaving</small> | <small>1.97</small> | <small>12.72</small> | <small>12</small> | <small>48</small> | <small>52</small> | <small>112</small> | <small>20.16</small> | <small> </small> | <small> </small> | <small>862</small> | <small>3.6</small> |
| <small>60</small> | <small>2</small> | <small> </small> | <small>100% cotton Combed</small> | <small>100%</small> | <small>Ring/Ring DbldTfo</small> | <small>weaving</small> | <small>To be updated</small> | <small>11.23</small> | <small>23</small> | <small>67</small> | <small>58</small> | <small>148</small> | <small>15.66</small> | <small> </small> | <small> </small> | <small>1035</small> | <small>4.8</small> |
| <small>60</small> | <small>2</small> | <small> </small> | <small>100% cotton Combed</small> | <small>100%</small> | <small>Ring/Ring DbldTfo</small> | <small>Gassed n merserised</small> | <small>To be updated</small> |  |  |  |  | <small> </small> | <small> </small> | <small> </small> | <small>18.16</small> | <small> </small> | <small> </small> | <small>776</small> | <small>3.6</small> |
| <small>56</small> | <small>2</small> | <small> </small> | <small>100% cotton Combed</small> | <small>100%</small> | <small>Ring/Ring DbldTfo</small> | <small>weaving</small> | <small>1.89</small> | <small>10.53</small> | <small>11</small> | <small>51</small> | <small>34</small> | <small>96</small> | <small>21.00</small> | <small> </small> | <small> </small> | <small>750</small> | <small>3.6</small> |
| <small>50</small> | <small>1</small> | <small> </small> | <small>100% cotton Combed</small> | <small>100%</small> | <small>Ring</small> | <small>Weaving n Knitting</small> | <small>1.82</small> | <small>12.01</small> | <small>31</small> | <small>189</small> | <small>167</small> | <small>387</small> | <small>17.12</small> | <small> </small> | <small> </small> | <small>1058</small> | <small>3.8</small> |
| <small>36</small> | <small>12</small> | <small> </small> | <small> </small> | <small> </small> | <small>Spec will be given on requst.</small> |  |  |  |  |  |  |  |  | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>40</small> | <small>1</small> | <small> </small> | <small>100% cotton Combed</small> | <small>100%</small> | <small>Ring</small> | <small>Weaving n Knitting</small> | <small>1.67</small> | <small>10.69</small> | <small>22</small> | <small>38</small> | <small> </small> | <small> </small> | <small>14.23</small> | <small> </small> | <small> </small> | <small>847</small> | <small>3.4</small> |
| <small>30</small> | <small>1</small> | <small> </small> | <small>100% cotton Combed</small> | <small>100%</small> | <small>Ring</small> | <small>Knitting</small> | <small>1.61</small> | <small>10.33</small> | <small>4</small> | <small>34</small> | <small>67</small> | <small>105</small> | <small>14.12</small> | <small> </small> | <small> </small> | <small>733</small> | <small>3.4</small> |
| <small>24</small> | <small>1</small> | <small> </small> | <small>100% cotton karded</small> | <small>100%</small> | <small>Ring</small> | <small>Weaving</small> | <small>1.72</small> | <small>12.52</small> | <small>24</small> | <small>155</small> | <small>189</small> | <small>368</small> | <small>13.97</small> | <small> </small> | <small> </small> | <small>810</small> | <small>4.2</small> |
| <small>20</small> | <small>1</small> | <small> </small> | <small>100% cotton karded</small> | <small>100%</small> | <small>Open End</small> | <small>Weaving</small> | <small>2.12</small> | <small>10.5</small> | <small>2</small> | <small>45</small> | <small>16</small> | <small>63</small> | <small>10.65</small> | <small> </small> | <small> </small> | <small>880</small> | <small>5</small> |
| <small>20</small> | <small>3x3</small> | <small> </small> | <small>Indutrial</small> | <small> </small> | <small> </small> | <small>Spec will be given on requst.</small> |  |  |  |  |  |  |  |  | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>20</small> | <small>8x3</small> | <small> </small> | <small>Indutrial</small> | <small> </small> | <small> </small> | <small>Spec will be given on request</small> |  |  |  |  |  |  |  |  | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>16</small> | <small>1</small> | <small> </small> | <small></small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>14</small> | <small>1</small> | <small> </small> | <small>weaving</small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>14</small> | <small>2 x2</small> | <small> </small> | <small>Indutrial</small> | <small> </small> | <small> </small> | <small>Spec will be given on request</small> |  |  |  |  |  |  |  |  | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>12</small> | <small>1</small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small>14-10%</small> |  |  | <small> </small> |
| <small>12</small> | <small>4x3</small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small>Spec will be given on request</small> |  |  |  |  |  |  |  |  | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>10</small> | <small>1</small> | <small> </small> | <small>Weaving/Knit Fleece</small> |  |  |  |  | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>10</small> | <small>8</small> | <small> </small> | <small>Fringe</small> | <small> </small> | <small> </small> | <small>Spec will be given on request</small> |  |  |  |  |  |  |  |  | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>10</small> | <small>6</small> | <small> </small> | <small>Filter cloth</small> | <small> </small> | <small> </small> | <small>Spec will be given on request</small> |  |  |  |  |  |  |  |  | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>PC COUNTS</small> <small></small><small></small><small></small><small></small><small></small><small></small><small></small><small></small><small></small><small></small><small></small><small></small><small></small><small></small><small></small><small></small> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| <small>70</small> | <small>2</small> | <small> </small> | <small>Poly cotton weaving</small> | <small>65/35</small> | <small>Ring/Ring DbldTfo</small> | <small>Weaving</small> | <small>1.81</small> | <small>9.34</small> | <small>0</small> | <small>17</small> | <small>22</small> | <small>39</small> | <small>24.63</small> | <small> </small> | <small> </small> | <small>862</small> | <small>3.7</small> |
| <small>60</small> | <small>2</small> | <small> </small> | <small>Poly cotton weaving</small> | <small>65/35</small> | <small>Ring/Ring DbldTfo</small> | <small>Weaving</small> | <small>1.73</small> | <small>9.19</small> | <small>0</small> | <small>14</small> | <small>29</small> | <small>43</small> | <small>27.10</small> | <small> </small> | <small> </small> | <small>906</small> | <small>4.2</small> |
| <small>50</small> | <small>1</small> | <small> </small> | <small>Poly cotton combed High twist </small> | <small>80/20</small> | <small>Ring</small> | <small>Weaving</small> | <small>1.87</small> | <small>12</small> | <small>22</small> | <small>100</small> | <small>167</small> | <small>289</small> | <small>24.50</small> | <small> </small> | <small> </small> | <small>1141</small> | <small>4.1</small> |
| <small>45</small> | <small>1</small> | <small> </small> | <small>Poly cotton weaving</small> | <small>65/35</small> | <small>Ring</small> | <small>Weaving</small> | <small>2.01</small> | <small>13</small> | <small>39</small> | <small>279</small> | <small>150</small> | <small>468</small> | <small>18.19</small> | <small> </small> | <small> </small> | <small>1083</small> | <small>4.1</small> |
| <small>40</small> | <small>1</small> | <small> </small> | <small>Poly cotton Knits</small> | <small>20/80 </small> | <small>Ring </small> | <small>Knitting</small> | <small>1.76</small> | <small>11.27</small> | <small>12</small> | <small>98</small> | <small>142</small> | <small>252</small> | <small>16.00</small> | <small> </small> | <small> </small> | <small>896</small> | <small> </small> |
| <small>40</small> | <small>2</small> | <small> </small> | <small>Poly cotton we aving</small> | <small>65/35 </small> | <small> </small> | <small>Weaving</small> | <small>1.56</small> | <small>9.91</small> | <small>1 </small> | <small>27</small> | <small>14 </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small>739 </small> | <small> </small> |
| <small>30</small> | <small>1</small> | <small> </small> | <small>Poly cotton weaving</small> | <small>65/35</small> | <small>Ring</small> | <small> </small> | <small>1.8</small> | <small> </small> | <small>36</small> | <small>203</small> | <small>247</small> | <small>486</small> | <small>22.12</small> | <small> </small> | <small> </small> | <small>884</small> | <small> </small> |
| <small>30</small> | <small>1</small> | <small> </small> | <small>Poly Cotton Karded</small> | <small>52/48</small> | <small>Ring</small> | <small>Knitting</small> | <small>1.53</small> | <small>12.8</small> | <small>40</small> | <small>189</small> | <small>167</small> | <small>396</small> | <small>17.10</small> | <small> </small> | <small> </small> | <small>755</small> | <small>3.5</small> |
| <small>30</small> | <small>1</small> | <small> </small> | <small>Poly Cotton combed</small> | <small>52/48</small> | <small>Ring</small> | <small>Knitting</small> | <small>1.54</small> | <small>10.5</small> | <small>0</small> | <small>45</small> | <small>50</small> | <small>95</small> | <small>18.10</small> | <small> </small> | <small> </small> | <small>690</small> | <small>3.2</small> |
| <small>30</small> | <small>1</small> | <small> </small> | <small>Poly cotton </small> | <small>20/80</small> | <small>Ring</small> | <small>Knitting</small> | <small>1.36</small> | <small>11</small> | <small>0</small> | <small>98</small> | <small>58</small> | <small>156</small> | <small>16.33</small> | <small> </small> | <small> </small> | <small>776</small> | <small>3.6</small> |
| <small>30</small> | <small>2</small> | <small> </small> | <small>Poly Cotton Karded</small> | <small>52/48</small> | <small>Ring/Ring DbldTfo</small> | <small>Weaving</small> | <small>1.65</small> | <small>9.73</small> | <small>0</small> | <small>29</small> | <small>52</small> | <small>81</small> | <small>23.00</small> | <small> </small> | <small> </small> | <small>579</small> | <small>3.8</small> |
| <small>30</small> | <small>1</small> | <small> </small> | <small>Poly Cotton combed</small> | <small>65/35</small> | <small>Ring</small> | <small>Knitting</small> | <small>1.52</small> | <small>10.39</small> | <small>0</small> | <small>39</small> | <small>42</small> | <small>81</small> | <small>19.81</small> | <small> </small> | <small> </small> | <small>690</small> | <small>3.2</small> |
| <small>24</small> | <small>1</small> | <small> </small> | <small>Poly Cotton Karded</small> | <small>52/48</small> | <small>Ring</small> | <small>Knitting</small> | <small>1.92</small> | <small>13.12</small> | <small>35</small> | <small>180</small> | <small>156</small> | <small>371</small> | <small>18.90</small> | <small> </small> | <small> </small> | <small>675</small> | <small>3.5</small> |
| <small>20</small> | <small>1</small> | <small> </small> | <small>Poly Cotton Karded</small> | <small>52/48</small> | <small>Open End</small> | <small>Weaving</small> | <small>2.13</small> | <small>11.3</small> | <small>3</small> | <small>31</small> | <small>16</small> | <small>50</small> | <small>14.66</small> | <small> </small> | <small> </small> | <small>757</small> | <small>4.3</small> |
| <small>16</small> | <small>1</small> | <small> </small> | <small>weaving</small> | <small>52/48</small> | <small>Open End</small> |  |  | <small>2.12</small> | <small>10.78</small> | <small>2</small> | <small>42</small> | <small>21</small> | <small>65</small> | <small>15.68</small> | <small> </small> | <small> </small> | <small>677</small> | <small>4.3</small> |
| <small>16</small> | <small>3</small> | <small> </small> | <small>Jumbo cones</small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>14</small> | <small>1</small> | <small> </small> | <small>Poly Cotton Karded</small> | <small>65/35</small> | <small>Open End</small> | <small>Weaving</small> | <small>2.1</small> | <small>10.93</small> | <small>3</small> | <small>39</small> | <small>18</small> | <small>60</small> | <small>18.70</small> | <small>2880</small> | <small>9.31</small> | <small>589</small> | <small>4</small> |
| <small>14</small> | <small>3</small> | <small> </small> | <small>Jumbo cones</small> | <small>65/35</small> |  |  | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>14</small> | <small>2 x 3 </small> |  |  |  |  |  |  | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>12</small> | <small>1</small> | <small> </small> | <small>weaving</small> | <small>65/35</small> |  |  | <small>Weav/Industrial</small> | <small>2.05</small> | <small>12.8</small> | <small>9</small> | <small>65</small> | <small>38</small> | <small>112</small> | <small>17.75</small> | <small>2750</small> | <small> </small> | <small> </small> | <small> </small> |
| <small>12</small> | <small>1</small> | <small> </small> | <small>weaving</small> | <small>52/48</small> |  |  | <small>Weav/Industrial</small> | <small>2.08</small> | <small>12.84</small> | <small>11</small> | <small>79</small> | <small>42</small> | <small>132</small> | <small>17.25</small> | <small>2450</small> | <small>13.5</small> | <small> </small> | <small> </small> |
| <small>12</small> | <small>3</small> | <small> </small> | <small>Jumbo cones</small> | <small>65/35</small> |  |  | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>12</small> | <small>5</small> | <small> </small> | <small>Jumbo cones</small> | <small>65/35</small> |  |  | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>9</small> | <small>1</small> | <small> </small> | <small>weaving</small> | <small>52/48</small> |  |  | <small> </small> | <small>2.1</small> | <small>12.3</small> | <small>8</small> | <small>56</small> | <small>9</small> | <small>73</small> | <small>16.70</small> | <small>2650</small> | <small>11</small> | <small> </small> | <small> </small> |
| <small>8</small> | <small>1</small> | <small> </small> | <small>weaving</small> | <small>65/35</small> |  |  | <small> </small> | <small>2.15</small> | <small>12.6</small> | <small>10</small> | <small>86</small> | <small>20</small> | <small>116</small> | <small>17.10</small> | <small>2775</small> | <small>10.5</small> | <small> </small> | <small> </small> |
| <small>7</small> | <small>1</small> | <small> </small> | <small>weaving</small> | <small>52/48</small> |  |  | <small> </small> | <small>2.13</small> | <small>12.7</small> | <small>22</small> | <small>97</small> | <small>26</small> | <small>145</small> | <small> </small> | <small>2600</small> | <small> </small> | <small> </small> | <small> </small> |
| <small>6</small> | <small>1</small> | <small> </small> | <small>weaving</small> | <small>65/35</small> |  |  | <small> </small> | <small>2.08</small> | <small>12.1</small> | <small>11</small> | <small>96</small> | <small>29</small> | <small>136</small> | <small>17.40</small> | <small>2900</small> | <small>11.8</small> | <small> </small> | <small> </small> |
| <small>6</small> | <small>1</small> | <small> </small> | <small>weaving</small> | <small>52/48</small> |  |  | <small> </small> | <small>2.1</small> | <small>12.9</small> | <small>19</small> | <small>100</small> | <small>22</small> | <small>141</small> | <small> </small> | <small>2670</small> | <small> </small> | <small> </small> | <small> </small> |
| <small>6</small> | <small>2x3</small> | <small> </small> | <small>Fringe </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>6</small> | <small>6</small> | <small>On Request </small> |  |  |  |  |  |  | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>PV COUNTS </small><small> </small><small> </small><small></small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| <small>60</small> | <small>2</small> | <small> </small> | <small>Poly Viscose </small> | <small>65/35</small> | <small>Ring/Ring DbldTfo</small> | <small>Weaving</small> | <small>1.93</small> | <small>9.48</small> | <small>0</small> | <small>19</small> | <small>42</small> | <small>61</small> | <small>25.17</small> | <small> </small> | <small> </small> | <small>712</small> | <small>3.3</small> |
| <small>45</small> | <small>2</small> | <small> </small> | <small>Poly Viscose </small> | <small>65/35</small> | <small>Ring/Ring DbldTfo</small> | <small>Weaving</small> | <small>1.56</small> | <small>9.19</small> | <small>0</small> | <small>10</small> | <small>43</small> | <small>53</small> | <small>27.04</small> | <small> </small> | <small> </small> | <small>616</small> | <small>3.3</small> |
| <small>30</small> | <small>1</small> | <small> </small> | <small>Poly Viscose </small> | <small>65/35</small> | <small>Ring</small> | <small>Weaving/ Knitting</small> | <small>1.61</small> | <small>9.43</small> | <small>2</small> | <small>24</small> | <small>76</small> | <small>102</small> | <small>23.01</small> | <small> </small> | <small> </small> | <small>647</small> | <small>3</small> |
| <small>28</small> | <small>2</small> | <small> </small> | <small>Poly Viscose </small> | <small>65/35</small> | <small>Ring/Ring DbldTfo</small> | <small>Weaving</small> | <small>1.59</small> | <small>8.91</small> | <small>0</small> | <small>11</small> | <small>21</small> | <small>32</small> | <small>22.32</small> | <small> </small> | <small> </small> | <small>486</small> | <small>3.3</small> |
| <small>100 % Spun Viscose</small> <small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| <small>30</small> | <small>1</small> | <small> </small> | <small>100 % Viscose</small> | <small>100%</small> | <small>Ring</small> | <small>Weaving/Knitting</small> | <small>1.97</small> | <small>12.27</small> | <small>9</small> | <small>45</small> | <small>67</small> | <small>121</small> | <small>15.27</small> | <small> </small> | <small> </small> | <small>638</small> | <small>2.96</small> |
| <small>40</small> | <small>2</small> | <small> </small> | <small>100 % Viscose</small> | <small>100%</small> | <small>Ring/Ring DbldTfo</small> | <small>Weaving</small> | <small>1.65</small> | <small>8.72</small> | <small>0</small> | <small>16</small> | <small>12</small> | <small>28</small> | <small>16.61</small> | <small> </small> | <small> </small> | <small>599</small> | <small>3.4</small> |
| <small>30</small> | <small>2</small> | <small> </small> | <small>100 % Viscose</small> | <small>100%</small> | <small>Ring/Ring DbldTfo</small> | <small>Weaving</small> | <small>1.59</small> | <small>8.29</small> | <small>0</small> | <small>9</small> | <small>10</small> | <small>19</small> | <small>17.81</small> | <small> </small> | <small> </small> | <small>518</small> | <small>3.4</small> |
| <small> </small><small> </small><small></small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| <small>Viscose cotton</small> <small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| <small>20</small> | <small>1</small> | <small> </small> | <small>Viscose Cotton</small> | <small>55/45</small> | <small>Spun</small> | <small>knitting</small> | <small>1.83</small> | <small>10.1</small> | <small>0</small> | <small>21</small> | <small>10</small> | <small>31</small> | <small>11.23</small> | <small> </small> | <small> </small> | <small>810</small> | <small>4.6</small> |
| <small>30</small> | <small>1</small> | <small> </small> | <small> </small> | <small>55/45</small> |  |  | <small>knitting</small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>40</small> | <small>1</small> | <small> </small> | <small> </small> | <small>67/33</small> |  |  | <small>knitting/weaving</small> |  |  |  |  | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small></small><small></small><small></small><small></small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small></small><small> </small> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| <small>100% Spun Polyester </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small><small> </small> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| <small>14</small> | <small>1</small> | <small> </small> | <small> </small> | <small> </small> | <small>Open End</small> |  |  | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>30</small> | <small>1</small> | <small> </small> | <small>100% Polyester</small> | <small>100%</small> | <small>Ring</small> <small></small> |  |  | <small>2.28</small> | <small>11.32</small> | <small>2</small> | <small>7</small> | <small>11</small> | <small>20</small> | <small>35.79</small> | <small> </small> | <small> </small> | <small>658</small> | <small>3.05</small> |
| <small>50</small> | <small>2</small> | <small> </small> | <small>100% Polyester</small> | <small>100%</small> | <small>Ring/Tfo DD</small> |  |  | <small>2.17</small> | <small>10.79</small> | <small>0</small> | <small>4</small> | <small>6</small> | <small>10</small> | <small>36.45</small> | <small> </small> | <small> </small> | <small>679</small> | <small>3.45</small> |
| <small>60</small> | <small>2</small> | <small> </small> | <small>100% Polyester</small> | <small>100%</small> | <small>Ring/Tfo DD</small> |  |  | <small>1.98</small> | <small>11.11</small> | <small>0</small> | <small>9</small> | <small>14</small> | <small>23</small> | <small>35.59</small> | <small> </small> | <small> </small> | <small>744</small> | <small>3.45</small> |
| <small>80</small> | <small>2</small> | <small> </small> | <small>100% Polyester</small> | <small>100%</small> | <small>Ring/Tfo DD</small> |  |  | <small>2.21</small> | <small>11.32</small> | <small>2</small> | <small>15</small> | <small>24</small> | <small>41</small> | <small>34.04</small> | <small> </small> | <small> </small> | <small>859</small> | <small>3.45</small> |
| <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>100% Polyester Sewing Threads -Made from super high tenacity polyester</small> <small> </small><small> </small><small> </small><small> </small> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> | <small> </small> |
| <small>60</small> | <small>3</small> | <small> </small> | <small>100% Polyester</small> | <small>100%</small> | <small>Ring/Tfo DD</small> | <small>Sewing</small> | <small>1.7</small> | <small>9.02</small> | <small>0</small> | <small>0</small> | <small>3</small> | <small>3</small> | <small>36.72</small> | <small> </small> | <small> </small> | <small>687</small> | <small>3.9</small> |
| <small>60</small> | <small>2</small> | <small> </small> | <small>100% Polyester</small> | <small>100%</small> | <small>Ring/Tfo DD</small> | <small>Sewing</small> | <small>1.89</small> | <small>9.31</small> | <small>0</small> | <small>2</small> | <small>4</small> | <small>6</small> | <small>35.12</small> | <small> </small> | <small> </small> | <small>798</small> | <small>3.7</small> |
| <small>50</small> | <small>2</small> | <small> </small> | <small>100% Polyester</small> | <small>100%</small> | <small>Ring/Tfo DD</small> | <small>Sewing</small> | <small>1.4</small> | <small>8.92</small> | <small>0</small> | <small>6</small> | <small>7</small> | <small>13</small> | <small>36.01</small> | <small> </small> | <small> </small> | <small>728</small> | <small>3.7</small> |
| <small>45</small> | <small>2</small> | <small> </small> | <small>100% Polyester</small> | <small>100%</small> | <small>Ring/Tfo DD</small> | <small>Sewing</small> | <small>1.98</small> | <small>9.1</small> | <small>0</small> | <small>5</small> | <small>6</small> | <small>11</small> | <small>35.00</small> | <small> </small> | <small> </small> | <small>691</small> | <small>3.7</small> |
| <small>40</small> | <small>2</small> | <small> </small> | <small>100% Polyester</small> | <small>100%</small> | <small>Ring/Tfo DD</small> | <small>Sewing</small> | <small>1.99</small> | <small>9.19</small> | <small>0</small> | <small>4</small> | <small>6</small> | <small>10</small> | <small>35.74</small> | <small> </small> | <small> </small> | <small>651</small> | <small>3.7</small> |
| <small>30</small> | <small>2</small> | <small> </small> | <small>100% Polyester</small> | <small>100%</small> | <small>Ring/Tfo DD</small> | <small>Sewing</small> | <small>1.95</small> | <small>8.97</small> | <small>0</small> | <small>4</small> | <small>7</small> | <small>11</small> | <small>35.21</small> | <small> </small> | <small> </small> | <small>564</small> | <small>3.7</small> |
| <small>30</small> | <small>3</small> | <small> </small> | <small>100% Polyester</small> | <small>100%</small> | <small>Ring/Tfo DD</small> | <small>Sewing</small> | <small>1.53</small> | <small>8.23</small> | <small>0</small> | <small>0</small> | <small>3</small> | <small>3</small> | <small>34.60</small> | <small> </small> | <small> </small> | <small>486</small> | <small>3.9</small> |
| <small>24</small> | <small>2</small> | <small> </small> | <small>100% Polyester</small> | <small>100%</small> | <small>Ring/Tfo DD</small> | <small>Sewing</small> | <small>1.96</small> | <small>8.94</small> | <small>0</small> | <small>3</small> | <small>2</small> | <small>5</small> | <small>37.60</small> | <small> </small> | <small> </small> | <small>505</small> | <small>3.7</small> |
| <small>20</small> | <small>2</small> | <small> </small> | <small>100% Polyester</small> | <small>100%</small> | <small>Ring/Tfo DD</small> | <small>Sewing</small> | <small>1.89</small> | <small>8.53</small> | <small>0</small> | <small>3</small> | <small>3</small> | <small>6</small> | <small>38.12</small> | <small> </small> | <small> </small> | <small>461</small> | <small>3.7</small> |
| <small>20</small> | <small>3</small> | <small> </small> | <small>100% Polyester</small> | <small>100%</small> | <small>Ring/Tfo DD</small> | <small>Sewing</small> | <small>1.68</small> | <small>8.21</small> | <small>0</small> | <small>0</small> | <small>2</small> | <small>2</small> | <small>40.12</small> | <small> </small> | <small> </small> | <small>396</small> | <small>3.9</small> |